Assignment 1

Q1. Write a Program(GPIO_A1) and Configure Pins PD13/PE8 , PD14/PE9

to LED3, LED4 Respectively & Toggle the LED's connected to them , also configure SWD Feature?

Q2. Write a Program(GPIO_A2) and Configure Pins PD12/PE8 to LED1 &

Set the LED's connected to the PIN for 500ms and Reset for 500ms , also configure SWD Feature?

Q3. Write a Program(GPIO_A3) and Configure Pins

(Switch)PA0  to GPIO_Input ,

(LED)   PD12/PE8 to GPIO_Output,

Action:Toggle the LED's when Switch is Pressed , also configure SWD Feature?

Part 1

/* USER CODE BEGIN WHILE */

while (1)

{

if(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_SET)

{

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_12);

}

}

Part 2

/* USER CODE BEGIN WHILE */

while (1)

{

if(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_SET)//

{

HAL_Delay(100); //Debouncing Delay used to avoid Noise

if(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_SET)//

{

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_12);

}

}

Q4. Write a Interrupt Driven Program(GPIO_A4) and Configure Pins

(Switch)PA0  to GPIO_EXTI0 ,

(LED)   PE8/PD12 to GPIO_Output,

NVIC GUI --> Enable EXTI line0 Interrupt

Action:Toggle the LED's when Switch is Pressed , also configure SWD Feature?

Part 1

/* USER CODE BEGIN PV */

uint8_t flag=0;

/* USER CODE BEGIN WHILE */

while (1)

{

if(1 == flag)

{

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_12);

flag=0;

}

/* USER CODE BEGIN 4 */

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

flag=1;

}

Part 2

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

HAL_GPIO_TogglePin(GPIOD, GPIO_PIN_12);

}

Flow

H/W Specific        HAL Specific

Switch(EXTI0 Source)--> EXTI0_IRQHandler --> HAL_GPIO_EXTI_IRQHandler() -->

Actual Funcion where we can Code HAL_GPIO_EXTI_Callback()

void EXTI0_IRQHandler(void)

{

/* USER CODE BEGIN EXTI0_IRQn 0 */

/* USER CODE END EXTI0_IRQn 0 */

HAL_GPIO_EXTI_IRQHandler(GPIO_PIN_0);

/* USER CODE BEGIN EXTI0_IRQn 1 */

/* USER CODE END EXTI0_IRQn 1 */

}

void HAL_GPIO_EXTI_IRQHandler(uint16_t GPIO_Pin)

{

/* EXTI line interrupt detected */

if(__HAL_GPIO_EXTI_GET_IT(GPIO_Pin) != 0x00u)

{

__HAL_GPIO_EXTI_CLEAR_IT(GPIO_Pin);

HAL_GPIO_EXTI_Callback(GPIO_Pin);

}

}

__weak void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

/* Prevent unused argument(s) compilation warning */

UNUSED(GPIO_Pin);

/* NOTE: This function should not be modified, when the callback is needed,

the HAL_GPIO_EXTI_Callback could be implemented in the user file

*/

}